	Instructions 
	Posizionamento


	[image: image1.jpg]sSide View (Green)

o

Stern — — -

GREEN
Parallel to the Horizont



[image: image2.jpg]Side View (Req)

— —|— Stern

RED
Parallel 1o the Horizont





	[image: image3.jpg]SIEPS





Position the basket, insert the cable, make the electric connection (brown cable to the positive terminal, blue cable to the negative terminal) and fix the light by the proper screw (N°2) supplied together with the assembly kit
[image: image4.jpg]T &
e v




Finally, insert the screen (N°1).



	[image: image5.jpg]\/

Parallel to the Horizont
( ) & &

Stem

- S’rern





Use gasket (N°5) to locate the drilling holes on the craft
[image: image6.jpg]23 P

" Stem «e - - o -& - Stern/





Make two holes  4mm (Position A and position C) and one hole  2,5 mm (position B)


	


	[image: image7.png]



	LED NAVIGATION LIGHTS   




Orionis side lights 112.5°

Side Lights for boats having length inferior or equal to 20mt

EPLF0002  GREEN
EPLF0003 RED
RINA TYPE APPROVAL CERTIFICATE No. ELE453105CS

INSTALLER’S 

OPERATING MANUAL
SIC DIVISIONE ELETTRONICA srl
WWW.SIC-DIVISIONE-ELETTRONICA.IT

LECCE – ITALY
Description:
Navigation lights for pleasure boats with length L<20  metres and approved in accordance with international standards on prevention of collision in the sea COLREG 1972
Side Lights 
[image: image8.jpg]


  [image: image9.jpg]



Positive (+) : Brown cable
Negative (-): Blue cable
	1. Screen made of polycarbonate o 
2. Screw 3X16 
3. Polycarbonate transparent dome
4. supply cable 2x22AWG 
5. Gasket 
	Technical Characteristics 
· Field of application: boats < 20m

· Visibility arc: 112.5°

· Input voltage: 9-30Vcc
· Luminous source: power red or green LED 1W

· Protected against polarity inversion and spike
· Equipped with supply cable
· Maximum absorbed power: 2W

· Impact resistant
· Mechanical protection standard:  IP56

· Average life of the LED: 10.000 hours
· Easy to install



	Placement and installation of the Navigation Lights
	

	[image: image10.jpg]Fron’r View

GREEN





	[image: image11.jpg]Top View







	[image: image12.jpg]Front View

GREEN




	[image: image13.jpg]Top View






	· The navigation lights shall be visible in the normal conditions of use.

· The navigation lights shall not be obscured by any fixed structures or by any optional equipment in the normal conditions of use.

· The navigation lights shall be installed in a way not to be a nuisance to the eyes of the operator.

· The navigation lights shall be installed in a way to ensure that the light appears in the required visibility arc and that is compliant with the required vertical separation and the placement requirements when the craft is in a static floating position 
	· The navigation lights shall be electrically connected to only one switch that operates all of the navigation lights together. Another switch used as a commutator shall switch on only the anchor light (360°). 

· The navigation lights shall be assembled in a way to reduce to the minimum any damages caused by contacts with other objects in normal conditions of use



www.sic-divisione-elettronica.it                                                                       






Rev. 450.2.09.06

